How to use SSCOM3.2?

Software SSCOM3.2 is used to change & check parameter of device. It is easy to use and really efficient if you could connect it to PC.

Parameters enquiry like: APN, Server, interval, status, time zone, gprsset etc.

Its method is making command like: AT^GT_CM=GPRSSET.

Detail steps are as following:

- Connect download cable with PC, and then open software SSCOM3.2.

- Choose the a ComNum and change Raudra be 115200, and click “Open Com”  and “EXT” like:

- After this set, please edit

- Data editable should be AT^GT_CM=”commands”, commands should be the command we send to device, but without #.

Like changing server platform to www.tracksolid.com is editing AT^GT_CM=SERVER,1,gpsdev.tracksolid.com,21100,0

And checking GPRSSET : AT^GT_CM=GPRSSET etc.

- Now connect download cable with device and turn device on.

- After it shows “Power on” at left blank space, click the number at the right side related with the one you want to change.

For example, if want to change its server to be www.tracksolid.com then click number 2.

- When you do not use it, please close the software first and then disconnect download cable.